Safety Data Sheet according to Regulation (EC) No. 1907/2006 (REACH) Page 1/11 Mycoplasma Off™ Revision date: 21 Aug 2024 Version: 5 Print date: 21 Aug 2024

Safety Data Sheet according to Regulation (EC) No. 1907/2006 (REACH)

SECTION 1: Identification of the substance/mixture and of the company/undertaking

- Product identifier Trade name/designation: Mycoplasma Off™ Article No.: 15-5000, 15-1000, 15-0050 UFI: XF6K-E0M5-UP1H-N57C - Relevant identified uses of the substance or mixture and uses advised against Use of the substance/mixture: The product is intended for research, analysis and scientific education. Relevant identified uses: Life cycle stage [LCS] PW: Widespread use by professional workers Sector of uses [SU] SU 24: Scientific research and development Product Categories [PC] PC 8: Biocidal product PC 35: Washing and cleaning products - Details of the supplier of the safety data sheet Supplier (manufacturer/importer/only representative/downstream user/distributor): Minerva Biolabs GmbH Schkopauer Ring 13 12681 Berlin Germany Telephone: +49 30 2000437 0 E-mail: ehs@minerva-biolabs.com Website: www.minerva-biolabs.com - Emergency telephone number Vergiftungs-Informations-Zentrale Freiburg, 24h: +49 761 19240

SECTION 2: Hazards identification

- Classification of the substance or mixture Classification according to Regulation (EC) No 1272/2008 [CLP] Hazard classes and hazard categories; Hazard statements; Classification procedure. flammable liquids (Flam. Liq. 3); H226: Flammable liquid and vapour.; On basis of test data.. Serious eye damage/eye irritation (Eye Dam. 1); H318: Causes serious eye damage.; Calculation method.. STOT-single exposure (STOT SE 3); H336: May cause drowsiness or dizziness.; Calculation method.. - Label elements Labelling according to Regulation (EC) No. 1272/2008 [CLP] Hazard pictograms: GHS02 GHS05 GHS07 Flame Corrosion Exclamation mark Signal word: Danger

en / DE

Safety Data Sheet according to Regulation (EC) No. 1907/2006 (REACH) Page 2/11 Mycoplasma Off™ Revision date: 21 Aug 2024 Version: 5 Print date: 21 Aug 2024

Hazard components for labelling: propan-1-ol Hazard statements for physical hazards. H226; Flammable liquid and vapour.. Hazard statements for health hazards. H318; Causes serious eye damage.. H336; May cause drowsiness or dizziness.. Supplemental hazard information: none Precautionary statements Prevention. P210; Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking.. P280; Wear protective gloves/protective clothing and eye protection/face protection.. Precautionary statements Response. P303 + P361 + P353; IF ON SKIN (or hair): Take off immediately all contaminated clothing. Rinse skin with water [or shower].. P304 + P340; IF INHALED: Remove person to fresh air and keep comfortable for breathing.. P305 + P351 + P338; IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing.. 2.3. Other hazards Adverse physicochemical effects: In case of insufficient ventilation and/or through use, explosive/highly flammable mixtures may develop. Adverse human health effects and symptoms: This product does not contain a substance that has endocrine disrupting properties with respect to humans as no components meets the criteria.

SECTION 3: Composition/information on ingredients

3.2. Mixtures Hazardous ingredients / Hazardous impurities / Stabilisers: Product identifiers; Substance name Classification according to Regulation (EC) No 1272/2008 [CLP]; Concentration. CAS No.: 71-23-8 EC No.: 200-746-9 Index No.: 603-003-00-0 REACH No.: 01-2119486761-29; propan-1-ol Eye Dam. 1 (H318), Flam. Liq. 2 (H225), STOT SE 3 (H336) Danger Acute Toxicity Estimate ATE (oral) 8,000 mg/kg ATE (dermal) 4,032 mg/kg ATE (inhalation, vapour) > 33.8 mg/L ATE (inhalation, dust/mist) > 51.91 mg/L; 30 – < 50 weight-%. CAS No.: 64-17-5 EC No.: 200-578-6 Index No.: 603-002-00-5 REACH No.: 01-2119457610-43-XXXX; ethanol Flam. Liq. 2 (H225) Danger Acute Toxicity Estimate ATE (oral) 10,470 mg/kg ATE (dermal) > 2,000 mg/kg ATE (inhalation, vapour) 116.9 mg/L; 10 – < 30 weight-%. CAS No.: 34590-94-8 EC No.: 252-104-2 REACH No.: 01-2119450011-60-XXXX; Dipropylene glycol monomethyl ether Substance with a community workplace exposure limit. Acute Toxicity Estimate ATE (oral) > 5,000 mg/kg ATE (dermal) 9,510 mg/kg; 1 – ≤ 5 weight-%. Full text of H- and EUH-phrases: see section 16.

SECTION 4: First aid measures

4.1. Description of first aid measures General information: In case of accident or unwellness, seek medical advice immediately (show directions for use or safety data sheet if possible). Remove victim out of the danger area. Remove contaminated, saturated clothing immediately. Do not leave affected person unattended. If unconscious but breathing normally, place in recovery position and seek medical advice. Warning First aider: Pay attention to self-protection!

en / DE

Safety Data Sheet according to Regulation (EC) No. 1907/2006 (REACH) Page 3/11 Mycoplasma Off™ Revision date: 21 Aug 2024 Version: 5 Print date: 21 Aug 2024

Following inhalation: Provide fresh air. In case of respiratory tract irritation, consult a physician. Get medical advice/attention if you feel unwell. In case of skin contact: After contact with skin, wash immediately with plenty of water and soap. Take off immediately all contaminated clothing. If skin irritation or rash occurs: Get medical advice/attention. After eye contact: In case of contact with eyes flush immediately with plenty of flowing water for 10 to 15 minutes holding eyelids apart and consult an ophthalmologist. Following ingestion: Rinse mouth. Get medical advice/attention if you feel unwell. Let 1 glass of water be drunken in little sips (dilution effect). Self-protection of the first aider: Use personal protection equipment. - Most important symptoms and effects, both acute and delayed Serious eye damage/eye irritation - Indication of any immediate medical attention and special treatment needed Treat symptomatically.

SECTION 5: Firefighting measures

- Extinguishing media Suitable extinguishing media: Water spray jet, alcohol resistant foam, Extinguishing powder, Carbon dioxide (CO2) Unsuitable extinguishing media: Full water jet - Special hazards arising from the substance or mixture Combustible. Vapours are heavier than air, spread along floors and form explosive mixtures with air. Hazardous combustion products: In case of fire: Carbon monoxide, Carbon dioxide (CO2). Vapours can form explosive mixtures with air. - Advice for firefighters Fight fire with normal precautions from a reasonable distance. Wear a self-contained breathing apparatus and chemical protective clothing. - Additional information Collect contaminated fire extinguishing water separately. Do not allow entering drains or surface water.

SECTION 6: Accidental release measures

- Personal precautions, protective equipment and emergency procedures - For non-emergency personnel Personal precautions: Remove persons to safety. Use personal protective equipment as required. Avoid contact with skin, eyes and clothes. Protective equipment: Wear protective gloves/protective clothing and eye protection/face protection. - For emergency responders Personal protection equipment: Personal protection equipment: see section 8 - Environmental precautions Do not allow to enter into surface water or drains. - Methods and material for containment and cleaning up For containment: Absorb with liquid-binding material (sand, diatomaceous earth, acid- or universal binding agents). For cleaning up: Water (with cleaning agent) - Reference to other sections Safe handling: see section 7, Personal protection equipment: see section 8, Disposal: see section 13 - Additional information Use appropriate container to avoid environmental contamination.

en / DE

Safety Data Sheet according to Regulation (EC) No. 1907/2006 (REACH) Page 4/11 Mycoplasma Off™ Revision date: 21 Aug 2024 Version: 5 Print date: 21 Aug 2024

SECTION 7: Handling and storage

- Precautions for safe handling Protective measures Advices on safe handling: Wear personal protection equipment (refer to section 8). Wash hands before breaks and after work. Keep away from: Food and feedingstuffs.If local exhaust ventilation is not possible or not sufficient, the entire working area should be ventilated by technical means. Do not breathe mist/vapours/spray. Fire prevent measures: Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking. Vapours can form explosive mixtures with air. Advices on general occupational hygiene Avoid contact with skin, eyes and clothes. When using do not eat, drink, smoke, sniff. - Conditions for safe storage, including any incompatibilities Technical measures and storage conditions: Keep container tightly closed in a cool, well-ventilated place. Keep out of reach of children. Requirements for storage rooms and vessels: Keep container tightly closed in a cool, well-ventilated place. Recommended storage temperature: at room temperature. Protect from sunlight. Hints on storage assembly: Do not store together with: Food and feedingstuffs (TRGS 510) Storage class (TRGS 510, Germany): 3 – Flammable liquids Further information on storage conditions: Keep container tightly closed in a cool, well-ventilated place. - Specific end use(s) No data available

SECTION 8: Exposure controls/personal protection

- Control parameters - Occupational exposure limit values Limit value type (country of origin); Substance name; ① Long-term occupational exposure limit value ② Short-term occupational exposure limit value ③ Instantaneous value ④ Monitoring and observation processes ⑤ Remark. TRGS 900 (DE) from 29 Mar 2019; ethanol CAS No.: 64-17-5 EC No.: 200-578-6; ① 200 ppm (380 mg/m³) ② 800 ppm (1,520 mg/m³) ⑤ DFG, Y. IOELV (EU); Dipropylene glycol monomethyl ether CAS No.: 34590-94-8 EC No.: 252-104-2; ① 50 ppm (308 mg/m³) ⑤ (may be absorbed through the skin). TRGS 900 (DE); Dipropylene glycol monomethyl ether CAS No.: 34590-94-8 EC No.: 252-104-2; ① 50 ppm (310 mg/m³) ② 50 ppm (310 mg/m³) ⑤ (Aerosol und Dampf) DFG, EU, 11. - Biological limit values No data available - DNEL-/PNEC-values No data available - Exposure controls - Appropriate engineering controls If local exhaust ventilation is not possible or not sufficient, the entire working area should be ventilated by technical means. Avoid breathing vapours and spray.

en / DE

Safety Data Sheet according to Regulation (EC) No. 1907/2006 (REACH) Page 5/11 Mycoplasma Off™ Revision date: 21 Aug 2024 Version: 5 Print date: 21 Aug 2024

- Personal protection equipment Eye/face protection: Eye glasses with side protection (EN 166) Skin protection: Tested protective gloves must be worn (EN ISO 374). Take recovery periods for skin regeneration. Breakthrough times and swelling properties of the material must be taken into consideration. Suitable material: Butyl caoutchouc (butyl rubber), Thickness of the glove material: > 0,35 mm. In the case of wanting to use the gloves again, clean them before taking off and air them well. Respiratory protection: Usually no personal respirative protection necessary. This product should not be used under conditions of poor ventilation unless a protective mask with an appropriate gas filter (i.e. type A1 according to standard EN 14387) is used. Other protection measures: Wash hands before breaks and after work. - Environmental exposure controls No data available

SECTION 9: Physical and chemical properties

- Information on basic physical and chemical properties Appearance Physical state: Liquid Colour: transparent Odour: Alcohol Safety relevant basis data Parameter; Value; at °C; ① Method ② Remark. pH; ≈ 8; 20 °C. Melting point; -114.5 °C; ① OECD 102. Freezing point; No data available. Initial boiling point and boiling range; No data available. Flash point; 28 °C; ① DIN 51755 part 1. Evaporation rate; No data available. Auto-ignition temperature; No data available. Upper/lower flammability or explosive limits; 2.5 – 15 Vol-%. Vapour pressure; No data available. Vapour density; No data available. Density; 0.9 g/cm³; 20 °C. Bulk density; not applicable. Water solubility; completely miscible. Dynamic viscosity; No data available. Kinematic viscosity; No data available. - Other information No data available

SECTION 10: Stability and reactivity

- Reactivity This material is considered to be non-reactive under normal use conditions. - Chemical stability The product is chemically stable under recommended conditions of storage, use and temperature. - Possibility of hazardous reactions No hazardous reaction when handled and stored according to provisions.

en / DE

Safety Data Sheet according to Regulation (EC) No. 1907/2006 (REACH) Page 6/11 Mycoplasma Off™ Revision date: 21 Aug 2024 Version: 5 Print date: 21 Aug 2024

- Conditions to avoid Further information on proper storage: see section 7. - Incompatible materials No further relevant information available. - Hazardous decomposition products No known hazardous decomposition products. Decomposition products in case of fire: see section 5.

SECTION 11: Toxicological information

- Information on hazard classes as defined in Regulation (EC) No 1272/2008 propan-1-ol CAS No.: 71-23-8 EC No.: 200-746-9. LD50 oral: 8,000mg/kg (Rat). LD50 dermal: 4,032mg/kg (Rabbit). LC50 Acute inhalation toxicity (vapour): >33.8mg/L 4h (Rat). LC50 Acute inhalation toxicity (dust/mist): >51.91mg/L 8h (rat) OECD Guideline 403 (Acute Inhalation Toxicity). ethanol CAS No.: 64-17-5 EC No.: 200-578-6. LD50 oral: 10,470mg/kg (Rat) OECD 401. LD50 dermal: >2,000mg/kg (Rat) OECD 402. LC50 Acute inhalation toxicity (vapour): 116.9mg/L 4h (Rat) OECD 403. Dipropylene glycol monomethyl ether CAS No.: 34590-94-8 EC No.: 252-104-2. LD50 oral: >5,000mg/kg (rat) OECD Guideline 401 (Acute Oral Toxicity). LD50 dermal: 9,510mg/kg (rabbit) OECD Guideline 402 (Acute Dermal Toxicity). Acute oral toxicity: Based on available data, the classification criteria are not met. Acute dermal toxicity: Based on available data, the classification criteria are not met. Acute inhalation toxicity: Based on available data, the classification criteria are not met. Skin corrosion/irritation: Based on available data, the classification criteria are not met. Serious eye damage/irritation: Causes serious eye damage. Causes serious eye damage. Respiratory or skin sensitisation: Based on available data, the classification criteria are not met. Germ cell mutagenicity: Based on available data, the classification criteria are not met. Carcinogenicity: Based on available data, the classification criteria are not met. Reproductive toxicity: Based on available data, the classification criteria are not met. STOT-single exposure: Based on available data, the classification criteria are not met. STOT-repeated exposure: Based on available data, the classification criteria are not met. Aspiration hazard: Based on available data, the classification criteria are not met. Additional information: No data available - Information on other hazards No data available

en / DE

Safety Data Sheet according to Regulation (EC) No. 1907/2006 (REACH) Page 7/11 Mycoplasma Off™ Revision date: 21 Aug 2024 Version: 5 Print date: 21 Aug 2024

SECTION 12: Ecological information

- Toxicity propan-1-ol CAS No.: 71-23-8 EC No.: 200-746-9. LC50: 4,555mg/L 4d (fish, Pimephales promelas). LC50: 4,555mg/L 4d (fish, Pimephales promelas Activated sludge). LC50: 4,555mg/L 4d (fish, Pimephales promelas) OECD Guideline 203 (Fish, Acute Toxicity Test). LC50: 1,000mg/L 2d (crustaceans, Gammarus pulex). EC50: >1,000mg/L (Belebtschlamm). EC50: 3,644mg/L 2d (fish, Daphnia magna). EC50: >1,000mg/L (Algae/water plant, Activated sludge). EC50: 9,170mg/L 2d (Algae/water plant, Raphidocelis subcapitata (previous names: Pseudokirchneriella subcapitata, Selenastrum capricornutum)). EC50: 3,644mg/L 2d (crustaceans, Daphnia magna) DIN 38412 Part 11, Daphnia- Short term test. NOEC: >100mg/L 21d (Algae/water plant, Daphnia magna). NOEC: 1,150mg/L 2d (Algae/water plant). NOEC: 68.3mg/L 21d (fish, Daphnia magna). NOEC: 1,150mg/L 2d (Algae/water plant, Chlorella pyrenoidosa). ethanol CAS No.: 64-17-5 EC No.: 200-578-6. LC50: 12,340mg/L 2d (Daphnia magna). LC50: 275mg/L 3d. LC50: 14,200mg/L 4d (fish, Pimephales promelas) US EPA method E03-05. LC50: 5,012mg/L 2d (crustaceans, Ceriodaphnia dubia) ASTM E729-80. EC50: 1,806mg/L (ceriodaphnia dubia, Chlorella vulgaris) OECD 201. EC50: 275mg/L 3d (Algae/water plant, Chlorella vulgaris) OECD Guideline 201 (Alga, Growth Inhibition Test). EC50: 675mg/L 4d (Algae/water plant, Chlorella vulgaris) OECD Guideline 201 (Alga, Growth Inhibition Test). EC50: 12,900mg/L 4d (fish, Pimephales promelas) US EPA method E03-05. NOEC: 4,432mg/L (ceriodaphnia dubia). NOEC: 2mg/L 10d (crustaceans, Ceriodaphnia dubia). Dipropylene glycol monomethyl ether CAS No.: 34590-94-8 EC No.: 252-104-2. LC50: >1,000mg/L 4d (fish, Poecilia reticulata). LC50: >1,000mg/L 2d (crustaceans, Crangon crangon) EPA OPP 72-3 (Estuarine/Marine Fish, Mollusk, or Shrimp Acute Toxicity Test). LC50: >1,000mg/L 3d (crustaceans, Crangon crangon) EPA OPP 72-3 (Estuarine/Marine Fish, Mollusk, or Shrimp Acute Toxicity Test). LC50: >1,000mg/L 4d (crustaceans, Crangon crangon) EPA OPP 72-3 (Estuarine/Marine Fish, Mollusk, or Shrimp Acute Toxicity Test). EC50: >969mg/L 3d (Algae/water plant, Raphidocelis subcapitata (previous names: Pseudokirchneriella subcapitata, Selenastrum capricornutum)). EC50: >969mg/L 4d (Algae/water plant, Raphidocelis subcapitata (previous names: Pseudokirchneriella subcapitata, Selenastrum capricornutum)). NOEC: 969mg/L 3d (Algae/water plant, Raphidocelis subcapitata (previous names: Pseudokirchneriella subcapitata, Selenastrum capricornutum)). NOEC: 969mg/L 4d (Algae/water plant, Raphidocelis subcapitata (previous names: Pseudokirchneriella subcapitata, Selenastrum capricornutum)). LOEC: 0.5mg/L 22d (crustaceans, Daphnia magna). - Persistence and degradability propan-1-ol CAS No.: 71-23-8 EC No.: 200-746-9. Biodegradation: Yes, rapidly. ethanol CAS No.: 64-17-5 EC No.: 200-578-6. Biodegradation: Yes, rapidly.

en / DE

Safety Data Sheet according to Regulation (EC) No. 1907/2006 (REACH) Page 8/11 Mycoplasma Off™ Revision date: 21 Aug 2024 Version: 5 Print date: 21 Aug 2024

- Bioaccumulative potential propan-1-ol CAS No.: 71-23-8 EC No.: 200-746-9. Log KOW: 0.2. Bioconcentration factor (BCF): 0.88. ethanol CAS No.: 64-17-5 EC No.: 200-578-6. Log KOW: -0.31. Bioconcentration factor (BCF): 3.2. Dipropylene glycol monomethyl ether CAS No.: 34590-94-8 EC No.: 252-104-2. Log KOW: 0.004. - Mobility in soil No data available - Results of PBT and vPvB assessment propan-1-ol CAS No.: 71-23-8 EC No.: 200-746-9. Results of PBT and vPvB assessment: This substance does not meet the PBT/vPvB criteria of REACH, Annex XIII.. ethanol CAS No.: 64-17-5 EC No.: 200-578-6. Results of PBT and vPvB assessment: This substance does not meet the PBT/vPvB criteria of REACH, Annex XIII.. Dipropylene glycol monomethyl ether; CAS No.: 34590-94-8; EC No.:; 252-104-2. Results of PBT and vPvB assessment: This substance does not meet the PBT/vPvB criteria of REACH, Annex XIII.. - Endocrine disrupting properties No data available - Other adverse effects No data available

SECTION 13: Disposal considerations

- Waste treatment methods Dispose of waste according to applicable legislation. - Product/Packaging disposal Waste codes/waste designations according to EWC/AVV Waste code product 07 06 04 * other organic solvents, washing liquids and mother liquors *: Evidence for disposal must be provided. Waste code packaging 15 01 02 Plastic packaging Waste treatment options Appropriate disposal / Product: Consult the appropriate local waste disposal expert about waste disposal. Appropriate disposal / Package: Completely emptied packages can be recycled.

SECTION 14: Transport information

Land transport (ADR/RID); Inland waterway craft (ADN); Sea transport (IMDG); Air transport (ICAO-TI / IATA- DGR). 14.1. UN number or ID number. UN 1987; UN 1987; UN 1987; UN 1987. 14.2. UN proper shipping name. ALCOHOLS, N.O.S. (propan-1- ol, ethanol); ALCOHOLS, N.O.S. (propan-1- ol, ethanol); ALCOHOLS, N.O.S. (propan-1- ol, ethanol); ALCOHOLS, N.O.S. (propan-1- ol, ethanol). 14.3. Transport hazard class(es). 3; 3; 3; 3. 14.4. Packing group. III; III; III; III.

en / DE

Safety Data Sheet according to Regulation (EC) No. 1907/2006 (REACH) Page 9/11 Mycoplasma Off™ Revision date: 21 Aug 2024 Version: 5 Print date: 21 Aug 2024

Land transport (ADR/RID); Inland waterway craft (ADN); Sea transport (IMDG); Air transport (ICAO-TI / IATA- DGR). 14.5. Environmental hazards. No; No; No; No. 14.6. Special precautions for user. Special Provisions: 274 ¦ 601 Limited quantity (LQ): 5 L Excepted Quantities (EQ): E1 Hazard identification number (Kemler No.): 30 Classification code: F1 Tunnel restriction code: (D/E); Special Provisions: 274 ¦ 601 Limited quantity (LQ): 5 L Excepted Quantities (EQ): E1 Classification code: F1; Special Provisions: 223 ¦ 274 Limited quantity (LQ): 5 L Excepted Quantities (EQ): E1 EmS-No.: F-E, S-D; Special Provisions: A3 ¦ A180 Limited quantity (LQ): Y344 Excepted Quantities (EQ): E1. 14.7. Maritime transport in bulk according to IMO instruments No transport as bulk according to IBC Code.

SECTION 15: Regulatory information

- Safety, health and environmental regulations/legislation specific for the substance or mixture - EU legislation Other regulations (EU): Hazard categories: - P5a Flammable Liquids, Category 1 or 2 - P5b Flammable liquids - P5c Flammable liquids of Categories 2 or 3, not covered by P5a and P5b 1907/2006 REACH, 1272/2008 CLP GHS, 98/24/EC, Regulation (EC) No. 648/2004 [Detergents regulation], KrW/AbfG - National regulations [DE] National regulations Restrictions of occupation 22 JArbSchG., 4 MuSchRiV., 5 MuSchRiV., Betriebssicherheitsverordnung (BetrSichV) Störfallverordnung (12. BImschV) for substances contained in the product: Hazard categories: - P5a Flammable Liquids, Category 1 or 2 - P5b Flammable liquids - P5c Flammable liquids of Categories 2 or 3, not covered by P5a and P5b for substances possibly developing during an incident: This product is not assigned to a hazard category. Betriebssicherheitsverordnung (BetrSichV) leichtentzündlich Water hazard class WGK: 1 - slightly hazardous to water Berufsgenossenschaftliche Vorschriften (DGUV-Vorschriften) Berufsgenossenschaftliche Vorschriften (DGUV-Vorschriften) Other regulations, restrictions and prohibition regulations Hazardous Substances Ordinance (GefStoffV) 15.2. Chemical Safety Assessment For this substance a chemical safety assessment has not been carried out.

SECTION 16: Other information

16.1. Indication of changes 1.3.; Details of the supplier of the safety data sheet. 2.2.; Label elements. 3.2.; Mixtures.

en / DE

Safety Data Sheet according to Regulation (EC) No. 1907/2006 (REACH) Page 10/11 Mycoplasma Off™ Revision date: 21 Aug 2024 Version: 5 Print date: 21 Aug 2024

4.1.; Description of first aid measures. 7.1.; Precautions for safe handling. 8.1.; Control parameters. 9.1.; Information on basic physical and chemical properties. 14.3.; Transport hazard class(es). 15.1.; Safety, health and environmental regulations/legislation specific for the substance or mixture. 16.1.; Indication of changes. 16.5.; List of relevant hazard statements and/or precautionary statements from sections 2 to 15. - Abbreviations and acronyms ACGIH American Conference of Governmental Industrial Hygienists ADN European Agreement concerning the International Carriage of Dangerous Goods by Inland Waterways ADR European Agreement concerning the International Carriage of Dangerous Goods by Road ASTM American Society for Testing and Materials BCF Bioconcentration Factor CAS Chemical Abstracts Service CLP Classification, Labelling and Packaging DIN German Institute for Standardization / German Industrial Standard DNEL derived no-effect level EC50 Effective Concentration 50% ECHA European Chemicals Agency EN European Standard ES Exposure scenario EWC European Waste Catalogue GHS Globally Harmonized System of Classification and Labelling of Chemicals IBC Intermediate Bulk Container ICAO International Civil Aviation Organization IMDG International Maritime Dangerous Goods IMO International Maritime Organization ISO International Standards Organisation LC50 Lethal (fatal) Concentration 50% LD50 Lethal (fatal) Dose 50% MAK Maximum concentration in the workplace air (CH) NFPA National Fire Protection Association NIOSH National Institute for Occupational Safety & Health NOEC No Observed Effect Concentration OECD Organisation for Economic Cooperation and Development OEL Threshold Limit Value OSHA Occupational Safety & Health Administration PBT persistent and bioaccumulative and toxic PC Product category PNEC Predicted No Effect Concentration REACH Registration, Evaluation and Authorization of Chemicals RID Dangerous goods regulations for transport by rail SU use category TRGS Technische Regeln für Gefahrstoffe UN United Nations For abbreviations and acronyms, see: ECHA Guidance on information requirements and chemical safety assessment, chapter R.20 (Table of terms and abbreviations). - Key literature references and sources for data TRGS 510, TRGS 525, TRGS 900, Safety data sheets for the ingredients, 1907/2006 REACH, 1272/2008 CLP GHS Substance name; Type; source of supply. propan-1-ol CAS No.: 71-23-8 EC No.: 200-746-9; LC50 Acute inhalation toxicity (dust/ mist); LC50; EC50; NOEC; Source: European Chemicals Agency, http://echa.europa.eu/. Dipropylene glycol monomethyl ether CAS No.: 34590-94-8 EC No.: 252-104-2; LD50 oral; LD50 dermal; LC50; EC50; NOEC; LOEC; Source: European Chemicals Agency, http://echa.europa.eu/. ethanol CAS No.: 64-17-5 EC No.: 200-578-6; LC50; EC50; NOEC; Source: European Chemicals Agency, http://echa.europa.eu/.

en / DE

Safety Data Sheet according to Regulation (EC) No. 1907/2006 (REACH) Page 11/11 Mycoplasma Off™ Revision date: 21 Aug 2024 Version: 5 Print date: 21 Aug 2024

- Classification for mixtures and used evaluation method according to regulation (EC) No 1272/2008 [CLP] Hazard classes and hazard categories; Hazard statements; Classification procedure. flammable liquids (Flam. Liq. 3); H226: Flammable liquid and vapour.; On basis of test data.. Serious eye damage/eye irritation (Eye Dam. 1); H318: Causes serious eye damage.; Calculation method.. STOT-single exposure (STOT SE 3); H336: May cause drowsiness or dizziness.; Calculation method.. - List of relevant hazard statements and/or precautionary statements from sections 2 to 15 Hazard statements. H225; Highly flammable liquid and vapour.. H318; Causes serious eye damage.. H336; May cause drowsiness or dizziness.. - Training advice No data available - Additional information This data sheet was created in accordance with EU regulation (EC) No. 1907/2006 (REACH). The information is based on the level of knowledge and experience on the date of issue, it do not have the importance of assured properties. The information may not be changed or transferred to other products. Reproduction in the unchanged state is permitted. * Data changed compared with the previous version.

en / DE